Your Attention Please!

A bug was found in IO.DLL that caused the error

"Unable to start the IO.SYS service" to occur on

Windows 2000/XP machines. This bug has been found

and was fixed on 08/06/02. If you are using an

older version of io.dll, you will want to upgrade

now!

Synopsis

IO.DLL allows seamless port I/O operations for

Windows 95/98/NT/2000/XP using the same library.

Introduction

In the pre-Windows days, it was a relatively

simple matter to access I/O ports on a typical PC.

Indeed, nearly every language sported a special

command for doing so. As Windows emerged and

gradually evolved, this flapping in the wind

behaviour could no longer be tolerated because of

operating system's ability to virtualize hardware.

Virtualizing hardware means that an application

(typically a DOS box in Windows) believes it is

talking directly to a physical device, but in

reality it is talking to a driver that emulates

the hardware, passing data back and forth as

appropriate. This is how you are able to open

dozens of DOS boxes in your Windows session, each

one with the strange notion that it has exclusive

access to peripherals such as the video adapter,

keyboard, sound card and printer.

If one were to rudely bang out data to an I/O port

that Windows thought it was in full control of,

the "official bad thing" could occur, the severity

of which depending upon the exact hardware that

was being accessed. Actually, with the

virtualization just mentioned, it is quite

improbable that Windows would permit anything too

nasty from occuring.

Windows 95/98 actually does allow I/O operations

be executed at the application level, although

you'd be hard pressed to find a language that

supports this directly. Typically the programmer

will have to resort to assembly language for this

kind of low-level control. If you know what you

are doing, this can be a quick and easy way to

access I/O ports. Of course, not everyone knows,

or desires to learn 80x86 assembly programming

just because they want to turn on a lamp from

their computer. However, the unwillingness to

learn assembly language becomes rather trivial

when faced with 9x's big brother.

Windows NT/2000/XP, being the secure operating

system that it is, does not permit port I/O

operations at the application level at all.

Period. A program with inline IN and OUT assembly

instructions that runs perfectly on Windows 95/98

will fail horribly when it comes to Windows

NT/2000/XP.

Windows NT/2000/XP does, however, allow I/O

instructions in its kernel mode drivers. A kernel

mode driver runs at the most priviledged level of

the processor and can do whatever it pleases,

including screwing up the system beyond repair,

thus writing a kernel mode driver is not for the

feint of heart.

If you were to take it upon yourself to wade

through the documentation of the Windows

NT/2000/XP ddk and piece together a driver that

was callable by your application to do the I/O

instructions on behalf of your application, you'd

probably notice something not too pleasant--this

sort of access is painfully slow. The call from

application level to system level typically takes

about one millisecond. Compare this to the one

microsecond that a normal I/O access takes. To

further the insult, you are at the whim of the

operating system. If it has tasks which it

believes are of higher priority than your lowly

call to your driver, it will perform them, making

precise timing nearly impossible.

Obviously, writing a driver that does acts a proxy

for the I/O calls isn't the most ideal solution.

There is, however, a solution for NT/2000/XP that

allows the same convienience of inline assembly

language that 95/98 does.

As mentioned, a kernel mode driver can do whatever

it wants. The implication here is that if another

kernel mode driver shut off application access to

the I/O ports, it should be possible for another

kernel mode driver to turn it back on. This is

where IO.DLL enters the picture.

Licensing

IO.DLL is completely free! However, you may not:

* Charge others for it in any way. For example,

you cannot sell it as a stand alone product.

* Claim that it is your property.

Also, the author (that's me) cannot be held liable

due to io.dll's failure to perform. As with most

free stuff, you are on your own.

Source Code and Special Modifications

The source code is available for $1,000 US.

I'm willing to work with people should they

require a special modification to IO.DLL. For

example, you might have a strict timing

requirement of some sort that can only be done in

kernel mode. For a fee, I will modify IO.DLL

and/or the embedded kernel mode driver for the

task at hand.

Description of IO.DLL

IO.DLL provides a useful set of commands for

reading and writing to the I/O ports. These

commands are consistent between 95/98 and

NT/2000/XP. Furthermore, there is no need for the

programmer to learn assembly language or muck with

kernel mode drivers. Simply link to the DLL and

call the functions. It's that easy.

Windows NT/2000/XP is accomodated through the use

of a small kernel mode driver that releases the

ports as needed to the application. This driver is

embedded in the DLL and is installed if Windows

NT/2000/XP is determined to be the underlying

operating system.

Due to the very minor overhead involved in

dynamically linking to IO.DLL, and the optimized

functions contained within, access to I/O ports is

nearly as fast as if it was written in raw

assembler and inlined in your application. This

holds true for both Windows 95/98 and Windows

NT/2000/XP.

Before moving on, it is probably prudent to

mention that the technique employed in IO.DLL for

releasing the ports to the application level

isn't, strictly speaking, the proper way to do

things. The proper way is to have a virtual device

driver for Windows 95/98 and a kernel mode driver

for Windows NT/2000/XP. This isn't very practical

for many people though, nor is it really

necessary. There are several successful commercial

products on the market that do exactly what IO.DLL

does. Let it be noted though that some of them are

shady with their explanation of how their product

works, meanwhile charging $500 or more for it.

Download

[45]io.dll 49k

C Prototypes

void PortOut(short int Port, char Data);

void PortWordOut(short int Port, short int Data);

void PortDWordOut(short int Port, int Data);

char PortIn(short int Port);

short int PortWordIn(short int Port);

int PortDWordIn(short int Port);

void SetPortBit(short int Port, char Bit);

void ClrPortBit(short int Port, char Bit);

void NotPortBit(short int Port, char Bit);

short int GetPortBit(short int Port, char Bit);

short int RightPortShift(short int Port, short int Val);

short int LeftPortShift(short int Port, short int Val);

short int IsDriverInstalled();

Delphi Prototypes

procedure PortOut(Port : Word; Data : Byte);

procedure PortWordOut(Port : Word; Data : Word);

procedure PortDWordOut(Port : Word; Data : DWord);

function PortIn(Port : Word) : Byte;

function PortWordIn(Port : Word) : Word;

function PortDWordIn(Port : Word) : DWord;

procedure SetPortBit(Port : Word; Bit : Byte);

procedure ClrPortBit(Port : Word; Bit : Byte);

procedure NotPortBit(Port : Word; Bit : Byte);

function GetPortBit(Port : Word; Bit : Byte) : WordBool;

function RightPortShift(Port : Word; Val : WordBool) : Word

Bool;

function LeftPortShift(Port : Word; Val : WordBool) : WordB

ool;

function IsDriverInstalled : Boolean;

Visual Basic Prototypes

Private Declare Sub PortOut Lib "IO.DLL" (ByVal

Port As Integer, ByVal Data As Byte)

Private Declare Sub PortWordOut Lib "IO.DLL"

(ByVal Port As Integer, ByVal Data As Integer)

Private Declare Sub PortDWordOut Lib "IO.DLL"

(ByVal Port As Integer, ByVal Data As Long)

Private Declare Function PortIn Lib "IO.DLL"

(ByVal Port As Integer) As Byte

Private Declare Function PortWordIn Lib "IO.DLL"

(ByVal Port As Integer) As Integer

Private Declare Function PortDWordIn Lib "IO.DLL"

(ByVal Port As Integer) As Long

Private Declare Sub SetPortBit Lib "IO.DLL" (ByVal

Port As Integer, ByVal Bit As Byte)

Private Declare Sub ClrPortBit Lib "IO.DLL" (ByVal

Port As Integer, ByVal Bit As Byte)

Private Declare Sub NotPortBit Lib "IO.DLL" (ByVal

Port As Integer, ByVal Bit As Byte)

Private Declare Function GetPortBit Lib "IO.DLL"

(ByVal Port As Integer, ByVal Bit As Byte) As

Boolean

Private Declare Function RightPortShift Lib

"IO.DLL" (ByVal Port As Integer, ByVal Val As

Boolean) As Boolean

Private Declare Function LeftPortShift Lib

"IO.DLL" (ByVal Port As Integer, ByVal Val As

Boolean) As Boolean

Private Declare Function IsDriverInstalled Lib

"IO.DLL" As Boolean

Function Descriptions

Please refer to the prototype for the particular

language you are using.

PortOut

Outputs a byte to the specified port.

PortWordOut

Outputs a word (16-bits) to the specified port.

PortDWordOut

Outputs a double word (32-bits) to the specified

port.

PortIn

Reads a byte from the specified port.

PortWordIn

Reads a word (16-bits) from the specified port.

PortDWordIn

Reads a double word (32-bits) from the specified

port.

SetPortBit

Sets the bit of the specified port.

ClrPortBit

Clears the bit of the specified port.

NotPortBit

Nots (inverts) the bit of the specified port.

GetPortBit

Returns the state of the specified bit.

RightPortShift

Shifts the specified port to the right. The LSB is

returned, and the value passed becomes the MSB.

LeftPortShift

Shifts the specified port to the left. The MSB is

returned, and the value passed becomes the LSB.

IsDriverInstalled

Returns non-zero if io.dll is installed and

functioning. The primary purpose of this function

is to ensure that the kernel mode driver for

NT/2000/XP has been installed and is accessible.

Notes

* When installing a new io.dll, it may be

necessary to first shut off the existing

io.sys kernel mode driver for NT/2000/XP. To

do this, open up a Command Prompt and enter

the command "net stop io.sys".